# CENIR CENTRE

### Original Configuration

- Display area: 18° × 18° DVA (square)

- Fixation grid: 5×5 grid (25 positions)

- Spacing: 4.5° DVA between positions

- Pursuit amplitudes: 3, 5, 7 DVA

- Image format: 800×800 pixels (square)

## New Configuration Summary

### Display Specifications

- Available aperture: 26.2° × 14.8° DVA (maximum)

- Chosen stimulus area: 22° × 12.375° DVA (rectangular, 16:9 aspect ratio)

- Display resolution: 1920×1080 pixels

- Pixels per DVA: ~73.3 pixels/DVA

### Stimulus Area Illustratiom

### Grid Specifications

- Layout: 7×3 grid (21 positions total)

- Spacing: 3.67° DVA (equal in both directions)

- Coverage: Full stimulus area (22° × 12.375° DVA)

- Centering: Perfectly centered at [0,0]

| Position | X (DVA) | Y (DVA) | Position | X (DVA) | Y (DVA) | Position | X (DVA) | Y (DVA) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -11.0 | +6.19 | 8 | -11.0 | 0.0 | 15 | -11.0 | -6.19 |
| 2 | -7.33 | +6.19 | 9 | -7.33 | 0.0 | 16 | -7.33 | -6.19 |
| 3 | -3.67 | +6.19 | 10 | -3.67 | 0.0 | 17 | -3.67 | -6.19 |
| 4 | 0.0 | +6.19 | 11 | 0.0 | 0.0 | 18 | 0.0 | -6.19 |
| 5 | +3.67 | +6.19 | 12 | +3.67 | 0.0 | 19 | +3.67 | -6.19 |
| 6 | +7.33 | +6.19 | 13 | +7.33 | 0.0 | 20 | +7.33 | -6.19 |
| 7 | +11.0 | +6.19 | 14 | +11.0 | 0.0 | 21 | +11.0 | -6.19 |

## Smooth Pursuit Task

### Amplitude Changes

- Original: 3, 5, 7 DVA

- Updated: 3, 5, 6 DVA

- Reason: Maximum amplitude reduced to ensure all 18 angles work within rectangular boundaries

### Angle Configuration

- Angles: All 18 original angles maintained (0°, 20°, 40°, ..., 340°)

- Boundary constraints:

  - Horizontal: ±11.0 DVA

  - Vertical: ±6.1875 DVA

- Validation: Automatic recomputation when movements exceed boundaries

### Movement Validation

Maximum safe movement envelope:

┌─────────────────────────────────┐

│                                                                         │ ±6.1875 DVA

│                                                                         │ (vertical limit)

│                                  ● (center)                     │

│                                                                         │

│                                                                         │

└─────────────────────────────────┘

±11.0 DVA (horizontal limit)

6 DVA amplitude is safe for all 18 angles

## Image Presentation Updates

### Image Specifications

- Original format: 800×800 pixels (square)

- New format: 1613×903 pixels (16:9 aspect ratio)

- Conversion method: AI-powered expansion (Canva Magic Expand)

- Display area: Full stimulus area (22° × 12.375° DVA)

### Image Processing Workflow

- Source: Square 800×800 images

- AI Extension: Expand to 1613×903 pixels using generative fill

- Aspect Ratio: 16:9 (matches stimulus area)

- Quality: No stretching or distortion of original content