# CENIR CENTRE

### Original Configuration

- Display area: 18° × 18° DVA (square)

- Fixation grid: 5×5 grid (25 positions)

- Spacing: 4.5° DVA between positions

- Pursuit amplitudes: 3, 5, 7 DVA

- Image format: 800×800 pixels (square)

## New Configuration Summary

### Display Specifications

- Available aperture: 26.2° × 14.8° DVA (maximum; full screen of 560 × 310 mm at 120 cm)

- Chosen stimulus area: 22° × 12.375° DVA nominal (rectangular, 16:9). The code converts both axes with the horizontal pixels per DVA, so the true vertical extent on the CENIR screen is ≈12.2°.

- Display resolution: 1920×1080 pixels

- Screen size: 560 × 310 mm

- Viewing distance: 120 cm

- Refresh rate: 120 Hz (videos recorded at 120 frames per second)

- Pixels per DVA: 71.8 px/DVA as used in the code (vaDeg2pix: 2 × 120 cm × tan(0.5°), with 1920 px over 560 mm), applied to both axes. True vertical value on this screen: 73.0 px/DVA.

### Stimulus Area Illustration

### Grid Specifications

- Layout: 7×3 grid (21 positions total)

- Spacing: 3.67° DVA horizontal (22 / 6), 6.19° DVA vertical (12.375 / 2)

- Coverage: Full stimulus area (22° × 12.375° DVA)

- Centering: Perfectly centered at [0,0]

- Numbering: row by row from the top-left (1 = top-left, 11 = centre, 21 = bottom-right); positive y = up.

| Position | X (DVA) | Y (DVA) | Position | X (DVA) | Y (DVA) | Position | X (DVA) | Y (DVA) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -11.0 | +6.19 | 8 | -11.0 | 0.0 | 15 | -11.0 | -6.19 |
| 2 | -7.33 | +6.19 | 9 | -7.33 | 0.0 | 16 | -7.33 | -6.19 |
| 3 | -3.67 | +6.19 | 10 | -3.67 | 0.0 | 17 | -3.67 | -6.19 |
| 4 | 0.0 | +6.19 | 11 | 0.0 | 0.0 | 18 | 0.0 | -6.19 |
| 5 | +3.67 | +6.19 | 12 | +3.67 | 0.0 | 19 | +3.67 | -6.19 |
| 6 | +7.33 | +6.19 | 13 | +7.33 | 0.0 | 20 | +7.33 | -6.19 |
| 7 | +11.0 | +6.19 | 14 | +11.0 | 0.0 | 21 | +11.0 | -6.19 |

## Smooth Pursuit Task

### Amplitude Changes

- Original: 3, 5, 7 DVA

- Updated: 3, 5, 6 DVA

- Reason: Maximum amplitude reduced so that a 6° movement starting from the centre stays within the ±6.1875° vertical limit at every angle

### Angle Configuration

- Angles: 18 angles in the design (0°, 20°, …, 340°; 0° = rightward, counter-clockwise, up = positive). Because each pursuit starts where the previous one ended, angles that would leave the window are redrawn at random. The angles are therefore not balanced: each appears 1–5 times per video, and some can be missing (80° does not occur in the 1.3 s video).

- Boundary constraints:

  - Horizontal: ±11.0 DVA

  - Vertical: ±6.1875 DVA

- Validation: Automatic recomputation when movements exceed boundaries

- Path: the first pursuit starts at the centre; each following pursuit starts at the previous end point and moves linearly over one TR; the last pursuit returns to the centre, whatever its logged angle.

### Movement Validation

Maximum safe movement envelope:

┌─────────────────────────────────┐

│                                                                         │ ±6.1875 DVA

│                                                                         │ (vertical limit)

│                                  ● (center)                     │

│                                                                         │

│                                                                         │

└─────────────────────────────────┘

±11.0 DVA (horizontal limit)

6 DVA is safe for all 18 angles only when starting from the centre; chained movements still trigger angle redraws.

## Image Presentation Updates

### Image Specifications

- Original format: 800×800 pixels (square)

- New format: 1613×903 pixels (16:9 aspect ratio)

- Conversion method: AI-powered expansion (Canva Magic Expand)

- Display area: full stimulus area, 22° × 12.375° DVA nominal = 1580 × 889 px on screen

### Image Processing Workflow

- Source: Square 800×800 images

- AI Extension: Expand to 1613×903 pixels using generative fill

- Aspect Ratio: 1.786 (1613 × 903), close to the 1.778 (16:9) of the stimulus area

- Quality: no distortion of the original content from the expansion. Scaling to 1580 × 889 px on screen compresses the image 0.5% more horizontally than vertically (negligible).

## Timing and Videos

### Trial Structure (one run)

- Pauses: 4 × 5 TR, target at the screen centre (before each part and at the end)

- Fixation: 42 trials × 1 TR (21 positions × 2, no immediate repeat)

- Smooth pursuit: 54 trials × 1 TR (3 amplitudes × 18 angles)

- Free viewing: 10 images × 3 TR (no target, cannot be used as gaze labels)

- Total: 146 TR per run; the scan must contain at least 146 volumes after the trigger

### Calibration Videos (1920 × 1080, 120 fps)

- TR 1.3 s: 22 776 frames, 189.8 s

- TR 1.46 s: 25 564 frames, 213.0 s

- TR 1.63 s: 28 598 frames, 238.3 s

- Video time: the video starts directly with trial 1 (no instruction screen); time of frame n = (n − 1) / 120 s. Each trial lasts round(duration in TR × TR × 120) frames.

- Event files: onset and duration in the video event files are raw Psychtoolbox clock times of a loop slowed by frame capture (≈5× too long). To be used only for the trial order, not for timing.